Table of contents

1	Table of contents	1

2	Introduction	2

2.1	Background	2

2.2	Purpose	2

2.3	Terms & definitions	2

3	Installation	3

4	Configuration	3

4.1	RSMPGS1.ini	3

4.2	SXL	4

5	Starting the simulator	4

6	Main Window	5

6.1	Sites and objects	5

6.2	System log	5

6.3	Generic	7

6.4	File	7

6.5	Process Image	8

6.6	Connection	8

6.7	RSMP	9

6.8	Alarms	10

6.9	Aggregated status	12

6.10	Status	13

6.11	Commands	14

6.12	Test send	14

7	Debug form	15

8	Change log	17

Introduction

This document describes the interface simulator for Road side systems which are communicating with RSMP.

Background

The Swedish Transport Administration has historically had a regional responsibility for the procurement and maintenance of technical systems. This has caused a non-homogenous view regarding functionality and interface issues in conjunction with supervision systems. Furthermore, the focus has been on the protocols of specific technical areas and suppliers, which has caused that there are limited opportunities to gather all systems into a national system. As a more national view has been established, the need for a national standard has arisen. This standard should be scalable, expandable and should have well defined interfaces between different levels.

Purpose

The purpose with RSMP is to create a standardized way for communication between systems on a local level and systems on a regional level, regardless of supplier and technical area. It should be possible to add and extract signals in new types of systems and applications without having to change the standard. This means that the protocol, in contrast to many other standards and protocols, should not contain detailed information about signal exchange. Instead, the aim is to define signal types which then are described according to specific systems or objects. The goal is that in the long term, based on installedsystems and objects, is to be able to produce signal exchange lists of type object that can be reused in newcontracts so that alarm messages, commands, etc. have the same names regardless of facility or provider.

The purpose of the signal exchange is to provide information relating to, for example, traffic control managersand administrators. E.g. the information needed to monitor and control the road side equipment, as well as the information that can be used for statistics and analysis of traffic and equipment status.

Terms & definitions

| Term | Description |
| --- | --- |
| HMI | Human Machine Interface (User interface) |
| SCADA | Supervisory Control And Data Acquisition |
| OP | Operating Panel |
| GUI | Graphical User Interface |
| RSMP | RoadSide Messaging Protocol |
| JSon | JavaScript Object Notation |
| TCP | Transfer Control Protocol |
| IP | Internet Protocol |
| GPRS | General Packet Radio Service |
| RSMP2Citect | Interface between roadside and Citect |
| RSMPGS1 | Interface simulator for Roadside Equipment |
| RSMPGS2 | Interface simulator for supervision system |

- Table 1. Terms& definitions

Installation

Installation should be with the file RSMPGS1_1_0_1_6_Setup.exe

The installation program recommends a suitable folder.

RSMPGS1 is written in C# with Microsoft Visual Studio 2015 and requires .NET Framework 4.6.1.

| Filename | Description |
| --- | --- |
| RSMPGS1.exe | Main program |
| .\Settings\RSMPGS1.INI | Configuration of RSMPGS1 |
| .\Objects | Reference files (example) |
| .\Source | Source code |
| .\Documents | Manual, RSMP spec etc |

Files and folders which are included in the RSMPGS1 installation. All files/maps are placed under the installation folder.

Configuration

RSMPGS1.ini

All base configuration is made in .\Settings\RSMPGS1.INI

The INI-file has the following parameters in the [RSMP] section:

IPAddress

IP-address and port number at the location of the supervision system, e. g 192.168.123.20:12666. The DNS name can also be used, e. gntsserver_2.trafikverket.se:12666

ReconnectInterval

Reconnection attempts will use this interval if automatic reconnection has been selected in the interface. Interval in milliseconds.

MaxDebugLines
Number of lines shown in the Debug-forms.

DaysToKeepLogFiles
Number of days before the log files are automatically deleted.

PacketTimeout

Packet timeout in milliseconds. The time which RSMPGS1 waits for ack/nack before an error message are written in the system log and the packet is resent.

WatchdogInterval
This interval will be used if cyclical watchdog has been selected in the user interface. The interval is in milliseconds.

WatchdogTimeout
The watchdog timeout in milliseconds, set to 0 to ignore failed watchdogs.

MaxEventsPerObject
Max number of events in the event views per object (oldest will be removed).

The INI-file has the following parameters in the [AggregatedStatus] section:

BitText_1..9
Texts which are associated (and presented in the user interface) in tab Aggregated Status.

The INI-file also has a [Main] section where for instance the current user interface configuration is saved.

SXL

RSMPGS1 supports both Excel-based SXL and YAML-based SXL.

SXL – CSV-format

RSMPGS2 cannot read the Excelformat which SXL is saved in. Every sheet of the SXL file (Excel) must be saved as a separate comma separated text file (CSV or SKV-file).

It is appropriate to name the file according to the content, e. g alarm.csv, 41101.csv, commands.csv etc. The files are saved in the sub-folder.\Objects. If the folder does not exist, it must be created. RSMPGS2 will create the folder automatically the first time the program is started. When the program is installed, several example files are included.

Do not forget to change or add a new revision number and save the first sheet again if any change is made. RSMPGS2 will point out if any file in .\Objects is changed without revision number change.

It is important that the SXLformat follows the template and the recommendations, otherwise RSMPGS2 won’t be able to read the information correctly. See the functional specification for more information on how to manually edit the CSV/SKV files if needed.

SXL – YAML-format

RSMPGS1 may import the YAML format found in GitHub – rsmp-nordic/sxl-tools. This format is more easy to read from code – yet still maintaining a human-redable format.

Starting the simulator

When RSMPGS1 is started it will automatically use the installation folder as the project folder. RSMPGS1 is expecting to find the Settings folder in the project folder.

If not already present, RSMPGS2 will create the LogFiles folders.

The program can also be started with the startparameter /path to use another project folder, e. g to use c:\RoadSide\TestConfig1 as a project folder:

RSMPGS1.EXE /path:c:\RoadSide\TestConfig1

In this way, it is possible to create shortcuts for an unlimited number of projects. These projects can also be run simultaneously.

The program will by default automatically try to find an Objects folder located in the project folder and scan it fo SXL (CSV) files, then loading them.

Main Window

The program has two forms, one main window and one (or several) debug windows.

At the top right, there is a presentation of connection status to the supervision system. If the box is green, the connection is ok. The IPaddress is also showing. When connection attempts are performed, the box is grey.

If there is no connection the box is red.

Sites and objects

At program startup the entire SXL is read from .\Objects and an hierarchical tree structure is built to the left. Clicking on an object group or object selects what should be presented on the tabs to the right.

When hovering over a node and ’Show all node info in Tooltip’ is selected, tooltip will show all information that RSMPGS1 have been able to read about respective objects.

System log

Down to the left, the system log is shown which display all occurring events, for instance if messages are received, if errors occur or a connection is established.

All events in the system log are continuously saved in ordinary text files in the folder  .\LogFiles\SysLogFiles and marked with today’s date. The files are automatically deleted when they are too old (according to settings in RSMPGS1.INI).

The System log will show different status icons, depending of the message itself (info, warning, error).

If View only failed packets is selected, the System log will only show failed packets.

Click Clear to empty the System log list. The System log files will not be deleted.

Generic

Various information concerning the connection to the supervision system is configured here.

When RSMPGS1 is connected to the supervision system, information about SXL and the RSMP-interface version is sent over for the software to determine whether communication is possible or not.

Active SXL (SUL) version to be used when connecting
SXL version which is sent over via the protocol when connection is made.

SXL (SUL) version found in file
SXL version which is found in reference files in .\Objects folder.

Always use SXL (SUL) version from file (if found)
Select to always use version number from the SXL files in protocol negotiation.

Automatically load last objects at startup
If selected RSMPGS1 will load the most recent used SXL object files/file at startup.

Automatically load process data at startup
Select to always load the last roadside data from ProcessImage.dat at startup

Automatically save process data on exit
Select to always save the last roadside data to  ProcessImage.dat when exiting RSMPGS1

When loading... Alarm status
Select to load last Alarm status from ProcessImage.dat during startup (if Automatically load process data at startup is selected)

When loading... Aggregated status values
Select to load last Aggregated status from ProcessImage.dat during startup (if Automatically load process data at startup is selected)

When loading... Status values 
Select to load last Status values from ProcessImage.dat during startup (if Automatically load process data at startup is selected)

Encryption protocols 
The RSMP protocol specification defines it is possible to use encryption (from version 3.1.3) based upon SSL 3.0 or TLS 1.0. As SSL 3.0 have been deprecated by the Internet Engineering Task Force (IETF) the simulator only supports the never TLS protocols.

Default will let the OS select the .NET version most suitable protocol.

The simulator does not look in the server's certificate store (beside to validate the CA), the certificate to use must be pointed out by the server. The idea is that the simulator should be used for testing a specific certificate.

To enable encryption it must be selected in the RSMP tab.

Server name
The server name is essential to validate the server certificate and is part of the negotiation process.

Ignore certificate errors
During the negotiation of the encrypted connection the system log will show some info. If there are any certificate errors the connection will be closed unless this choice is ticked.

Check certificate against certificate revocation list
Determines if the certificate should be checked against the revoced certificates and their CA's

Authenticate as client using this certificate file
If also the client should identify itself using a certificate it may be selected here using the Browse button. If it is a pfx-file the simulator will also ask for a password. Be aware of that the password is stored Base-64 encoded in the INI-file.

File

Load projects from – CSV-files

Select folder where the SXL-objects (CSV-files) reside

Load projects from – YAML-file

Select the YAML-file where the SXL-objects reside

Loading new objects is only available if the simulator is disconnected

Debug - Create new debug window

Creates a new debug window. Any number of debug forms can be created, but it will affect performance.

Debug -Tile all debug windows
Moves all debug forms to the right of RSMPGS1 (if there is space) and puts them in a line.

Debug - Close all debug windows

Closes all debug forms.

Debug - Store base64 updates

If checked, all incoming base64 encoded updates (commands) will be stored in the DebugFiles folder. The files will be named Base64_ followed by date/timestamp.

Exit
Closes RSMPGS1. Some process data is saved. Form settings (positions and what is marked) is saved in RSMPGS1.INI, so it looks the same at next program start.

Process Image

The process image contains information about status, commands and alarms.

Random update all subscriptions

If a client is subscribing to status updates, this selection can randomly change them. If the subscriptions are updated by events, the new values are sent directly up to the client otherwise they will be transmitted when it´s time according to UpdateRate.

The values of data type boolean, string and real will be updated to true/false, "0"/"1" and -1000.0..1000.0 respectively.  All other data types will be updated to -1000..1000.

Reset Alarm, Status, Aggregated and Command objects
Removes all alarms, status, aggregated status and command values. This can be done only when we are not connected.

Random update all Status values

Randomly change all Status values. This can be done only when we are not connected. Se data type value ranges above (Random update all subscriptions)

Save Process data to file...

Process data could be save to any file for later use. The process data file has a file ending of .dat but is basically a text file. The default process data file is the ProcessImage.dat file.

Load Process data from file...

Loads the process data file

Clear automatically saved process data

Removes the default process data file ProcessImage.dat.

Connection

Functions that are related to the supervision system connection is handled here. RSMPGS1 is acting as a server when it comes to providing data, alarms etc. But in connection terms, it is a client (TCP socket client).

The reason for this is that it´s easier to configure a firewall which is centrally located and where the supervision system likely is placed, rather than the opposite. Roadside equipment can consequently be situated behind a simpler firewall.

Connect automatically
Select to let RSMPGS1make automatic connection attempts to the supervision system. In RSMPGS1.INI, the interval for connection attempts are configured.

Connect now

Immediately performs a connection attempt, which is done regardless if ’connect automatically' has been marked or not.

Disconnect

Closes the connection. If ’connect automatically’is marked, a connection attempt will be performed within the stated interval.

Send options – Send watchdog packet now
Sends a watchdog package

Send options – Send some random crap
To test the resilience of supervision system regarding incoming junk data. It randomly produces 2048 bytes between 0x00..0xff. Just like an ordinary Json package it is terminated with 0xc0 (formfeed).

Send options – Disable Nagle algorithm (send coalescing)
This alternative affects the algorithm usually used in TCP to make the sending of many small packets more efficient. Ordinarily these are grouped together in larger packages.

RSMPGS1 is buffering all JSon packages with the C# function NetworkStream.Write() in two calls, where the first is the serialized data and the last is the packet termination character 0x0c (fromfeed).

If the algorithm is shut off, there will always be two packets out on the network. The purpose is, just like the next function, to test the buffer algorithm and packet decoding of supervision system.

Send options – Split packets
This alternative splits all packets randomly and sends them in small packets, 1..10 bytes each and 10 ms break between each packet. The purpose is to test the buffer algorithm and packet decoding, which are common error causes, and which may be hard to detect.

View

Always show group headers
If a parent node is selected in the Sites and Objects list view all its children will populate the listview in the selected tab (Alarms, Status, Commands). To separate them they will be grouped and there will be a group header. If only one child is selected by default the group header is not shown but will be shown if it is selected here.

Clear Alarm Events list
This will clear the alarm event list (it does not change any status)

RSMP

RSMP protocol specific settings have an own tab. The simulator could be used with RSMP protocol versions 3.1.1..3.1.5

Behaviour

The protocol behaviour could be adjusted to test different functionality. The settings could be changed  for each version of the RSMP protocol (not common settings). If any setting deviate from the default setting, it will be indicated by a red background.

The RSMP versions the simulator will allow and use when connecting are selected by the first row.

The setting Use strict and unforgiving protocol parsing enables a more strict mode, where amongst other protocol checking all JSon names and (where applicable) values are case-sensitive.

Each individual setting is not explained in this document, since they mostly reflects the version document history of the RSMP protocol and the protocol specification itself, hence are pretty much self-explanatory.

Connection statistics

Some statistics (sent bytes/packets etc) are viewed here. Select Clear to reset them.

Alarms

Alarms are read from SXL and created for each object type, which means the same AlarmCodeId may occur on several objects.

At the bottom, Timestamp and MsgId are shown for occurred events.

Select the alarm that should be tested and select from the pop-up menu with the right button.

Activate
Sets the alarm in active status (alternatively Inactive) and creates an Alarm-message with aSp == ’Issue’. The alarm automatically gets the status Acknowledge = false when activated.

Acknowledge
Sets the alarm in acknowledged status and creates an Alarm-message with aSp == ’Acknowledge’.

Suspend
Sets the alarm in suspended status (alternatively not suspended) and creates an Alarm-message with aSp == ’Suspend’.

Alarm Events

Alarm event history, Timestamp, MsgId, AlarmCodeId and Event are shown for sent and received events.

Aggregated status

Aggregated status is normally compiled on each grouped object of the road side equipment. RSMPGS1 lacks that kind of dynamic, the reason for this is that it is hard to keep track of which objects that are put in alarm mode. Consequently, status is configured manually.

Status bits
Double click to change bit status, on or off. Bit-texts are sourced from RSMPGS1.INI. The colors follow NTS standard.

Functional position
Click to choose a Functional Position. The empty field set Functional Position to null.

Functional state
Click to choose a Functional State. The empty field set Functional State to null.

Both FunctionalPosition and FunctionalState are read from SXL. Any ’-’ are automatically removed.

Send aggregated Status update
Send an ’AggregatedStatus’ message.

Automatically send update when anything is changed Status update
Select to send an ’AggregatedStatus’ message each time something has been changed.

Status

Status is sent to the supervision system after requests or cyclically/at changes when the supervision system is subscribing to them.

To change status, double click in the ’Status’column at the value that should be changed. At startup, a question mark is shown which will result in that the value which are sent up becomes null (unknown). If the value is manually changed, it will get the status ’recent’.

Values can be set to any value, all values are sent up as ’string’, which means that it can be tested how the supervision system reacts to invalid values.

If the data type is ’base64’, an extra button’Browse’ will be shown when double clicking ’status’ and a file can be selected, for example a bitmap.

Alternatively, the path to the file can be entered. If there is at least one ’\’ in the status field RSMPGS1 assumes that it is a file path/name and that the file should be base64-encoded and sent. If not, the field is sent as-is.

Please note that RSMPGS1/RSMPGS2 has limited buffer size and it cannot receive files larger than 2 MB.

In subscription mode, new status is sent directly when it has has changed if the subscription parameter UpdateRateis set to 0, in other cases when the interval expires next time.

Commands

The Commands tab displays the values which have been sent from the supervision system. At startup, only question marks are shown.

RSMPGS1 makes a basic check that the value is kept within the limits of the data type. If the value seems invalid, an error message is shown in the system log.

Test send

JSon is text based and in this tab text files can be sent as is. The files may be JSon debug data from the debug-forms.

There are two textboxes which can be used simultaneously. In the textboxes, it is possible to copy/paste text as desired.

Send above package
Sends Json message in textbox above. The ending 0x0c (formfeed) is automatically added at the end.

Please note that RSMPGS1 does not remove CR/LF or tabs in the text before it is sent.

Browse
Open and read a text file inte the above text form.

Buffered messages

When disconnected RSMPGS1 may buffer messages and send when reconnecting.

Create

Creates the selected message type and queue it. The number of messages to create may be set in the text box. RSMGS1 have no upper buffered message limit (memory only) but max 30’000 messages may be created per click on the Create button.

Buffered events may be deleted using the buttons to the right.

Buffered count

Indicates the total number of buffered events

Don't show these packets in system log if they are more than 10

If a lot of buffered messages are to be sent when connected, the system log will be flooded and take some time to fill. Ticking this box will significally improve speed.

Debug form

The Debug form shows sent and received data more or less formatted depending on chosen display format.

The red texts are messages which are received, green texts are the ones which are sent out. The time stamp has millisecond resolution which is shown if the column is expanded somewhat.

If show raw data is not selection, then RSMPGS1 will format the text for better readability. If there are CR/LF and tabs in the received Json, these control characters will be used to increase readability. If not, RSMPGS1 will use a set of simple rules to format the text.

RSMPGS1 is always removing CR/LF, tabs and finalizing form feed (FF) before deserializing.

Time stamps are also presented decoded in both UTC and local time.

Select what to debug – Show all traffic in raw format
Shows all packets unformatted in raw data format. ASCII codes which RSMPGS1 is not certain if they are writable are shown as <ASCII> i hex-format, e. g.<0x0c>forformfeed.

Select what to debug – All packet types
Shows all packet types.

Select what to debug –Version packets, Alarm Packets etc…
If not all packet types are selected to be shown, one packet type or a selection of packet types can be selected for display. Because that it is possible to display any number of debug formulas, it is possible to have different types of formulas for every packet type.

Copy selection to Clipboard
In the debug window, it is possible to select one or more lines and copy to the clipboard with this function. RSMPGS1 delimits every line with CR/LF (0x0d/0x0a) as text in the clipboard.

Every column is delimited with a tab (0x09).

Clear debug list
Empties the debug form.

Save continuous to file (record)…

Begins a recording to file from this specific debug window.

File name and file location are determined by the user. RSMPGS1 suggests the folder name\LogFiles\DebugFiles.

If the file does not exist it will be created and filled up with new debug data. The same data which is presented in the form is stored in the file. The file is an ordinary text file and will be flushed every 100 milliseconds. Because of this, it can be copied or opened (only for reading) with complete contents without having to be closed first.

To close the file and stop recording this option must be unchecked (it will be checked when the file is opened and recording is in progress).

Close debug form
Closes the debug window.

Change log

| Version | Document date | Change | Name |
| --- | --- | --- | --- |
| RevA | 2011-12-06 | First version of the document | Tomas Rook |
| RevB | 2012-08-24 | Change of template | Johan Thell |
| RevC | 2019-02-26 | Updated for new release of simulator | Tomas Rook |
| RevD | 2020-03-26 | Updated for new release of simulator | Tomas Rook |
| RevE | 2020-11-26 | Updated for new release of simulator | Tomas Rook |